# VIC-AMBHAS input files for the Philippine Hydrological Model

## Description of the Philippine Hydrological Model

The Philippine Hydrological Model is the first national-scale hydrological model of the Philippines. Its primary purpose is to quantify components of the hydrological cycle at the national level, with spatio-temporal patterns of precipitation, evapotranspiration, runoff and groundwater recharge as model outputs.

We have developed an integrated surface water-groundwater model based on the Variable Infiltration Capacity (VIC) macro-scale hydrological model (Liang et al. 1994, Hamman et al 2018), into which we have added a one-layer, 2D lateral groundwater flow model (Scheidegger et al. 2021). Groundwater recharge is derived from the interaction of the groundwater model with the VIC soil by allowing bi-directional exchange of water between the aquifer and the soil. The model is run at a 2 km grid resolution and is parameterised with, and driven by, globally available datasets describing the land surface, including soil and vegetation properties.

The model outputs include: evapotranspiration, runoff, groundwater recharge, baseflow, groundwater levels, and soil moisture.

To run this model the VIC-AMBHAS-GRID version of the code is used, accessed here: https://github.com/BritishGeologicalSurvey/VIC/blob/VIC-AMBHAS-GRID/vic/extensions/AMBHAS/readme.md

## Description of model structure

The Philippine Hydrological Model is split into four sub-models: North, Centre, South and West. These are referred to as LOCATION below. Each of these sub-models is run separately. Within each model folder, the following sub-folders are required:

• AMBHAS

• AMBHAS_OUT

• grid

• results

• run

The folders AMBHAS_OUT and results are empty at the start of the simulation. The contents of the other folders are described below.

## Description VIC input files

The VIC input files are stored in the folder run.

• VIC global parameters: VIC_global_params_LOCATION.txt. This is the main input file for VIC. It points VIC to the locations of other input/output files and sets parameters that govern the simulation, such as start and end dates.

• VIC parameter file: VIC_params_LOCATION.nc. Spatially distributed parameters describing the land surface.

• VIC domain file: VIC_Domain_LOCATION.nc. Domain information of VIC run.

A full description can be found here:

https://vic.readthedocs.io/en/master/Documentation/Drivers/Image/Inputs/

## Description of AMBHAS input file

The groundwater input files are stored in the folder AMBHAS.

• AMBHAS global parameter file for AMBHAS: gw_global_parameters.dat

• AMBHAS parameter file: gw_LOCATION_geol.nc

• AMBHAS observation points: gw_observation.dat

A full description of the input files can be found here:

https://github.com/BritishGeologicalSurvey/VIC/blob/VIC-AMBHAS-GRID/vic/extensions/AMBHAS/readme.md

## Description of Grid input files

The grid input files are stored in the folder grid.

• Grid global parameter file: grid_global_params.dat

• Grid forcing cell numbers: Force_CellPhilippines.nc

• VIC cell numbers: VIC_CellNo_LOCATION.nc

The inclusion of the grid modules allows the model to run using forcing data at its original spatial resolution and saves the user from re-gridding the climate data to the model extent and grid resolution. Each cell number from the forcing data, as specified in the Forcing cell numbers, then points to the corresponding VIC cell numbers, that maps the forcing cell onto the VIC grid.

A full description of the input files can be found here:

https://github.com/BritishGeologicalSurvey/VIC/blob/VIC-AMBHAS-GRID/vic/extensions/AMBHAS/readme.md

## Description of Forcing data

The input files containing the climate forcing data are stored in the folder forcing_ERA5.

One netcdf file per year is provided at a six hourly time step.

Descriptions of the variables and units can be found here: https://vic.readthedocs.io/en/master/Documentation/Drivers/Image/ForcingData/

## Link to the Code

VIC is written in the C programming language. The code can be found here:

https://github.com/BritishGeologicalSurvey/VIC/tree/VIC-AMBHAS-GRID

## Running the model on a Linux cluster

Advice on how to run and compile VIC can be found here:

https://vic.readthedocs.io/en/master/Documentation/Drivers/Image/RunVIC/

We run the model by using the batch script found in the run folder.

Data sources

Table 1. Description and data source of variables for the VIC global parameter file.

| Variable number | Variable name | Source | Description |
| --- | --- | --- | --- |
| 1 | mask | (Global Administrative Areas, 2012) | Country and Island outline |
| 2 | layer | - | - |
| 3 | run_cell | - | 1 = Run Grid cell, 0 = Do not Run |
| 4 | gridcell | - | Grid cell number |
| 5 | lats | - | Latitude |
| 6 | lons | - | Longitude |
| 7 | infilt | callibration parameter | Variable Infiltration parameter (binfilt). The binfilt parameter is the parameter used to describe the Variable Infiltration Curve. This is typically a value that is adjusted during the calibration of the VIC model. Parameter values range from 10-5 to 0.4. Higher values The b_infilt parameter is the parameter used to describe the Variable Infiltration Curve. This is typically a value that is adjusted during the calibration of the VIC model. Parameter values range from 10-5 to 0.4. Higher values will produce more runoff. 0.2 is often used as a starting value. |
| 8 | Ds | Dummy, not used | The soil parameter Ds represents the fraction of the Dsmax parameter at which non-linear base-flow occurs. This is typically a parameter that is adjusted during the calibration of the VIC model. An initial value of 0.001 may be used. Typically this value is small (less than 1). For the coupled VIC-AMBHAS model, this parameter is not used in the simulation, but needs to be given in the input files. |
| 9 | Dsmax | Dummy, not used | The parameter Dsmax is the maximum velocity of baseflow for each grid cell. This can be estimated using the saturated hydraulic conductivity, Ksat, for each grid cell multiplied by the slope of the grid cell. The values for Ksat can be averaged for the layers for which baseflow will be included. When working in decimal degrees, the elevation data for the basin should be projected to an equal area map projection, in order to have horizontal dimensions in the same units as the vertical dimensions so that the slopes computed in Arc/Info are meaningful lues. For the coupled VIC-AMBHAS model, this parameter is not used in the simulation, but needs to be given in the input files. |
| 10 | Ws | Dummy, not used | The parameter Ws is the fraction of maximum soil moisture where non-linear baseflow occurs. As with the Ds parameter, this is generally adjusted during the calibration phase of applying the VIC model. Values for Ws are typically greater than 0.5. An initial value of 0.9 can be used. For the coupled VIC-AMBHAS model, this parameter is not used in the simulation, but needs to be given in the input files. |
| 11 | c | Dummy, not used | c Exponent used in baseflow curve, normally set to 2. For the coupled VIC-AMBHAS model, this parameter is not used in the simulation, but needs to be given in the input files. |
| 12 | expt | Calculated from wilting point and field capacity by Zhang and Marcel (2018). https://dataverse.harvard.edu/dataset.xhtml?persistentId=doi:10.7910/DVN/UI5LCE The wilting point is given at h = 15000 cm, and the field capacity at h = 330 cm. b = slope of the retention curve in log – log space expt = 3 + 2*b | Exponent n (=3+2/lambda) in Campbells eqn for hydraulic conductivity, HBH 5.6 (where lambda = soil pore size distribution parameter). Values should be > 3.0 |
| 13 | Ksat | Zhang and Marcel (2018) https://dataverse.harvard.edu/dataset.xhtml?persistentId=doi:10.7910/DVN/UI5LCE | Saturated hydraulic conductivity mm/d |
| 14 | phi_s | Zhang and Marcel (2018) https://dataverse.harvard.edu/dataset.xhtml?persistentId=doi:10.7910/DVN/UI5LCE | Soil moisture diffusion parameter |
| 15 | init_moist | Initial soil moisture is set to porosity * layer depth | init_moist in mm Initial moisture content of each layer can be set at any reasonable value. One approach is to use fractional soil moisture content (expressed as a fraction of the maximum soil moisture; max. soil moisture = porosity * layer depth) at the critical point, Wcr, which can be computed for each layer as a depth in i meters by multiplying Wcr by the thickness of the layer in meters, and then multiplying by 1000 |
| 16 | elev | HydroSheds (Lehner et al 2008) | Average elevation of grid cell |
| 17 | depth |  | Thickness of each soil moisture layer. This is set to 0.1, 0.5, 2m below ground |
| 18 | avg_T | Fick and Hijmans (2017). https://www.worldclim.org/data/worldclim21.html | Average soil temperature. This parameter is the temperature of the soil at the damping depth. This temperature is often assumed to be the same as the average annual air temperature. This temperature is used as the bottom boundary of all thermal flux calculations made for the soil column. |
| 19 | dp | 4m | This is the soil thermal damping depth. It is defined as the depth in the soil column at which the soil temperature remains nearly constant annually. This is the depth to which soil thermal flux calculations will be made, and is often set to 4m. The constant temperature at this boundary is defined with the parameter avg_T. |
| 20 | bubble | bubble =0.32*expt + 4.3 | The bubble parameter is the bubbling pressure, h, for the soil texture type (see, e.g., Table 5.3.2 in Rawls, et al (Handbook of Hydrology)). This parameter is necessary for running the VIC model with FULL_ENERGY==TRUE or FROZEN_SOIL==TRUE. Values must be > 0.0. If you have a VIC soil parameter file created for water-balance mode runs, in which bubbling pressure has been set to a "nodata" value such as -99, you will not be able to use this soil parameter file for FULL_ENERGY==TRUE or FROZEN_SOIL==TRUE. However, a quick way to estimate bubbling pressure from the existing soil parameters (namely the expt parameter) is: bubble = 0.32*expt + 4.3 This is illustrated in figure 1, generated by taking the data from table 5.3.2 in Rawls, et al (Handbook of Hydrology), computing expt from lambda, and performing a linear regression. |
| 21 | quartz | SoilGrid1km variable SNDPPT(Hengl et al., 2014) | Quartz content of soil. Here the sand fraction is used. |
| 22 | bulk_density | SoilGrids1km BLDFIE (Hengl et al., 2014) | Bulk density of organic portion of soil. |
| 23 | soil_density | Soil particle density, normally 2685 kg/m3 | Soil particle density of organic portion of soil. |
| 24 | off_gmt | - | Time zone offset from GMT. |
| 25 | Wcr_FRACT | Calculated from Zhang using phi_s and the field capacity as follows: phi=phi_s*(0.7*FieldCapacity/psi_s)^(-1/b) Wcr_FRACT=phi/phi_s https://dataverse.harvard.edu/dataset.xhtml?persistentId=doi:10.7910/DVN/UI5LCE | The parameter Wcr_FRACT (Wcr) is the fractional soil moisture (expressed as a fraction of the maximum soil moisture; max. soil moisture = porosity * layer depth) at the critical point, which is the water content below which hydraulic conductivity begins to fall below saturated values, as does transpiration. This is set at 70% of the field capacity, in accordance with the different soil textures. Field Capacity is defined as the water content at a tension of -33kPa. |
| 26 | Wpwp_FRACT | Wilting point from Zhang and Marcel (2018) divided by phi_s from Zhang and Marcel (2018) https://dataverse.harvard.edu/dataset.xhtml?persistentId=doi:10.7910/DVN/UI5LCE | The parameter Wpwp_FRACT (wp) is the fractional soil moisture (expressed as a fraction of the maximum soil moisture; max. soil moisture = porosity * layer depth) at the wilting point. Wilting Point is set at the water content at a tension of 1500 kPa, and is approximated for the different soil textures. |
| 27 | rough | 0.001 | Surface roughness of bare soil, expressed in meters, can be set to a value 0.001, and adjusted according to local data. |
| 28 | snow_rough | 0 | The surface roughness of the snowpack, expressed in meters, can be set to an initial value of 0.0005, and can then be adjusted according to local data. |
| 29 | annual_prec | Fick and Hijmans (2017) https://www.worldclim.org/data/worldclim21.html | Average annual precipitation |
| 30 | resid_moist | Zhang and Marcel (2018). https://dataverse.harvard.edu/dataset.xhtml?persistentId=doi:10.7910/DVN/UI5LCE | Soil moisture layer residual moisture content in units of residual moisture content / total layer volume [mm/mm]. |
| 31 | fs_active | 0 | If set to 1, then frozen soil algorithm is activated for the grid cell. A 0 indicates that frozen soils are not computed even if soil temperatures fall below 0C |
| 32 | cellnum | same as gridcell | Grid cell number |
| 33 | AreaFract | 1 | Fraction of grid cell covered by each elevation band. Sum of the fractions must equal 1. |
| 34 | elevation | HydroSheds (Lehner et al 2008) | Mean (or median) elevation of elevation band. This is used to compute the change in air temperature from the grid cell mean elevation |
| 35 | Pfactor | 1 | Fraction of cell precipitation that falls on each elevation band. Total must equal 1. To ignore effects of elevation on precipitation, set these fractions equal to the area fractions |
| 36 | veg_descr | MODIS landcover at 0.05 °(Friedl and Sulla-Menashe, 2015). https://lpdaac.usgs.gov/products/mcd12c1v006/ | Land cover classification |
| 37 | Nveg | 1 | Number of vegetation tiles in the grid cell |
| 38 | Cv | 1 | Fraction of grid cell covered by vegetation tile |
| 39 | root_depth | Fan et al. (2017a) and dataset Fan et al. (2017b) https://wci.earth2observe.eu/thredds/catalog/usc/root-depth/catalog.html | Root zone thickness (sum of depths is total depth of root penetration) |
| 40 | root_fract | Calculated from root_depth | Fraction of root in the current root zone |
| 41 | LAI | Copernicus LAI at 1km (Smets et al., 2019) https://land.copernicus.eu/global/products/lai | Leaf Area Index, one per month |
| 42 | overstory | MODIS landcover at 0.05°(Friedl and Sulla-Menashe, 2015) and VIC vegetation library. https://lpdaac.usgs.gov/products/mcd12c1v006/ | Lag to indicate whether or not the current vegetation type has an overstory TRUE for overstory present [e.g. trees], FALSE for overstory not present [e.g. grass]) |
| 43 | rarc | MODIS landcover at 0.05°(Friedl and Sulla-Menashe, 2015) and VIC vegetation library. https://lpdaac.usgs.gov/products/mcd12c1v006/ | Architectural resistance of vegetation type (~2 s/m) Not sure about this!! #use the values from the veglib |
| 44 | rmin | MODIS landcover at 0.05°(Friedl and Sulla-Menashe, 2015) and VIC vegetation library. https://lpdaac.usgs.gov/products/mcd12c1v006/ | Minimum stomatal resistance of vegetation type (~100 s/m) use the values from the veglib |
| 45 | wind_h | MODIS landcover at 0.05°(Friedl and Sulla-Menashe, 2015) and VIC vegetation library. https://lpdaac.usgs.gov/products/mcd12c1v006/ | Height at which wind speed is measured |
| 46 | RGL | MODIS landcover at 0.05°(Friedl and Sulla-Menashe, 2015) and VIC vegetation library and VIC global parameter file. https://lpdaac.usgs.gov/products/mcd12c1v006/ | Minimum incoming shortwave radiation at which there will be transpiration. For trees this is about 30 W/m2, for crops about 100 W/m2. |
| 47 | rad_atten | default 0.5 | Radiation attenuation factor. Normally set to 0.5, though may need to be adjusted for high latitudes. |
| 48 | wind_atten | default 0.5 | Wind speed attenuation through the overstory. The default value has been 0.5. |
| 49 | trunk_ratio | default 0.2 | Ratio of total tree height that is trunk (no branches). The default value has been 0.2. |
| 50 | albedo | Copernicus surface albedo at 1km (Smets and Sánchez-Zapero, 2018). https://land.copernicus.eu/global/products/sa | Shortwave albedo for vegetation type |
| 51 | veg_rough | Vegetation roughness length is typically 0.123 * vegetation height. Vegetation height is obtained from Healey et al. (2015). https://webmap.ornl.gov/wcsdown/dataset.jsp?ds_id=10023 | Vegetation roughness length (typically 0.123 * vegetation height) one value per month |
| 52 | displacement | Vegetation displacement height is typically 0.67 * vegetation height. Vegetation height is obtained from Healey et al. (2015). https://webmap.ornl.gov/wcsdown/dataset.jsp?ds_id=10023 | Vegetation displacement height (typically 0.67 * vegetation height) one value per month |

Table 2. Parameters and sources for the Groundwater model coupled to VIC.

| Variable number | Variable name | Units | Description | Data source |
| --- | --- | --- | --- | --- |
| 1 | Sy | [-] | Specific_yield | Groundwater availability map of the Philippines parameterised (Bureau of Mines and Geo-Sciences and Ministry of Natural Resources, 1986). |
| 2 | Trans | [m2/day] | Transmissivity – a value must be given, even if K is specified | dummy |
| 3 | K | [m/day] | hydraulic_conductivity – a value must be given, even if T is spcified | Groundwater availability map of the Philippines parameterised (Bureau of Mines and Geo-Sciences and Ministry of Natural Resources, 1986). |
| 4 | mask | [-] | mask: active cells of the model domain are set to 1 | - |
| 5 | dem | [m] | digital elevation model | HydroSheds void filled DEM (Lehner et al 2008) |
| 6 | zbase | [m] | Base of the aquifer in m above datum | 100 m |
| 7 | zriver | [m] | elevation of the river elevation, e.g DEM or DEM - 5 | Dem -5 (Lehner et al 2008) |
| 8 | driver | [m] | Thickness of the river bed | 1 |
| 9 | C_eff | [1/day] | Conductance for leakage cells of effluent_river | Calibration, network based on HydroSheds flow accumulation (Lehner et al 2008) |
| 10 | C_in | [1/day] | Conductance for leakage cells of influent_river | Calibration, network based on HydroSheds flow accumulation (Lehner et al 2008) |
| 11 | C_leak_eff | [1/day] | Conductance for leakage cells effluent | 0 |
| 12 | C_leak_in | [1/day] | Conductance for leakage cells influent | 0 |
| 13 | headBC | [m] | Head[m] of specific head boundary cells, or -999 for non-specified head boundary nodes | 0 at the coast |
| 14 | river_area | [m2] | River area or cells where a river is present | Grid cell area |
| 15 | aquifer map | [-] | Zones of 1 are unconfined aquifer, and zones of 0 are confined aquifer | 1 |
| 16 | c_n | [m] | distance of the cell centre to the cell centre of the cell to the north [m] | Calculated distance |
| 17 | c_e | [m] | distance of the cell centre to the cell centre of the cell to the east [m] | Calculated distance |
| 18 | e_n | [m] | length of the northern edge of the cell [m] | Calculated distance |
| 19 | e_e | [m] | length of the eatern edge of the cell [m] | Calculated distance |
| 20 | cell_area | [m2] | cell area of each cell in [m2] | Calculated distance |
| 21 | z_soil | [m] | total soil depth used in the VIC model | 2 |
| 22 | Sy_soil | [m] | specific yield of the soil as used in the VIC model | Phi_s from Zhang and Marcel (2018) |

Table 3. Parameters and sources for the forcing file in VIC.

| Variable number | Variable name | Units | Description | Data source |
| --- | --- | --- | --- | --- |
| 1 | mask | [-] | mask |  |
| 2 | prcp | [mm/step] | Total precipitation | (Hersbach et al. 2018) |
| 3 | tas | [C] | Average air temperature | (Hersbach et al. 2018) |
| 4 | dswrf | [W/m2] | Incoming shortwave radiation | (Hersbach et al. 2018) |
| 5 | dlwrf | [W/m2] | Incoming longwave radiation | (Hersbach et al. 2018) |
| 6 | pres | [kPa] | Atmospheric pressure | (Hersbach et al. 2018) |
| 7 | vp | [kPa] | Vapor pressure | (Hersbach et al. 2018) |
| 8 | wind | [m/s] | Wind speed | (Hersbach et al. 2018) |

References

British Geological Survey: Philippine National Hydrological Model Dataset, NERC EDS National Geoscience Data Centre, https://doi.org/10.5285/9a8dffe7-5bf7-496c-9d0a-99dea86c631c, 2024.

Bureau of Mines and Geo-Sciences and Ministry of Natural Resources: Groundwater availability map of the Philippines, Bureau of Mines and Geo-Sciences, Geology and Mineral Resources of the Philippines. Volume two Mineral Resources. Plate supplement XVI,  Metro Manila, Philippines, 1986.

Fan, Y., Miguez-Macho, G., Jobbágy, E. G., Jackson, R. B., and Otero-Casal, C.: Hydrologic regulation of plant rooting depth, Proceedings of the National Academy of Sciences, 114, 10572, https://doi.org/10.1073/pnas.1712381114, 2017a.

Fan, Y., Miguez-Macho, G., Jobbágy, E. G., Jackson, R. B., and Otero-Casal, C.: Hydrologic regulation of plant rooting depth [dataset], 2017b.

Fick, S. E. and Hijmans, R. J.: WorldClim 2: new 1-km spatial resolution climate surfaces for global land areas, International Journal of Climatology, 37, 4302-4315, https://doi.org/10.1002/joc.5086, 2017.

Friedl, M. and Sulla-Menashe, D.: MCD12C1 MODIS/Terra+Aqua Land Cover Type Yearly L3 Global 0.05Deg CMG V006. 2015, distributed by NASA EOSDIS Land Processes DAAC [dataset], https://doi.org/10.5067/MODIS/MCD12C1.006, 2015.

Global Administrative Areas: GADM database of Global Administrative Areas, version 2.0.,  2012.

Hamman, J. J., Nijssen, B., Bohn, T. J., Gergel, D. R., and Mao, Y. X.: The Variable Infiltration Capacity model version 5 (VIC-5): infrastructure improvements for new applications and reproducibility, Geoscientific Model Development, 11, 3481-3496, https://doi.org/10.5194/gmd-11-3481-2018, 2018.

Healey, S. P., M.W. Hernandez, D.P. Edwards, M.A. Lefsky, E. Freeman, P.L. Patterson, E.J. Lindquist, and Lister., A. J.: CMS: GLAS LiDAR-derived Global Estimates of Forest Canopy Height, 2004-2008, https://daac.ornl.gov/cgi-bin/dsviewer.pl?ds_id=1271 [dataset], https://doi.org/10.3334/ORNLDAAC/1271, 2015.

Hengl, T., de Jesus, J. M., MacMillan, R. A., Batjes, N. H., Heuvelink, G. B., Ribeiro, E., Samuel-Rosa, A., Kempen, B., Leenaars, J. G., Walsh, M. G., and Gonzalez, M. R.: SoilGrids1km--global soil information based on automated mapping, PLoS One, 9, e105992, https://doi.org/10.1371/journal.pone.0105992, 2014.

Hersbach, H., Bell, B., Berrisford, P., Biavati, G., Horányi, A., Muñoz Sabater, J., Nicolas, J., Peubey, C., Radu, R., Rozum, I., Schepers, D., Simmons, A., Soci, C., Dee, D., and Thépaut, J.-N.: ERA5 hourly data on single levels from 1979 to present, Copernicus Climate Change Service (C3S) Climate Data Store (CDS) [dataset], https://doi.org/10.24381/cds.adbb2d47, 2018.

Lehner, B., Verdin, K., Jarvis, A. (2008). New global hydrography derived from spaceborne elevation data. Eos, Transactions, American Geophysical Union, 89(10): 93–94. https://doi.org/10.1029/2008eo100001.

Liang, X.; Lettenmaier, D. P.;   Wood, E. F. and  Burges, S. J. 1994: A simple hydrologically based model of land surface water and energy fluxes for general circulation models, J. Geophys. Res., 99(D7), 14415-14428, doi:10.1029/94JD00483.

Lohmann, D., Nolte-Holube, R., and Raschke, E.: A large-scale horizontal routing model to be coupled to land surface parametrization schemes, Tellus, Series A: Dynamic Meteorology and Oceanography, 48, 708-721, https://doi.org/10.3402/tellusa.v48i5.12200, 1996.

Scheidegger, Johanna. 2025 Future projections of the hydrology of the Philippines: dataset summary. Nottingham, UK, British Geological Survey, 28pp. (OR/25/010) (Unpublished).

Scheidegger, Johanna. 2024 User guide: BGS Philippine National Hydrological Model dataset. British Geological Survey, 45pp. (OR/24/023).

Scheidegger, J., Barkwith, A., Jackson, C., Mansour, M., and Guzman, A.: Philippine National Hydrological Model, British Geological Survey, 2023.

Scheidegger, J. M., Jackson, C. R., Muddu, S., Tomer, S. K., and Filgueira, R.: Integration of 2D Lateral Groundwater Flow into the Variable Infiltration Capacity (VIC) Model and Effects on Simulated Fluxes for Different Grid Resolutions and Aquifer Diffusivities, Water, 13, 663, https://doi.org/10.3390/w13050663, 2021.

Smets, B. and Sánchez-Zapero, J.: Copernicus Global Land Operations "Vegetation and Energy" ”CGLOPS-1”, Copernicus [dataset], 2018.

Smets, B., Verger, A., Camacho, F., Van der Goten, R., and Jacobs, T.: Copernicus Global Land Operations "Vegetation and Energy", Copernicus [dataset], https://land.copernicus.eu/global/products/lai, 2019.

Zhang, Y. and Marcel, G. S.: A High-Resolution Global Map of Soil Hydraulic Properties Produced by a Hierarchical Parameterization of a Physically-Based Water Retention Model (V1), Harvard Dataverse [dataset], https://doi.org/10.1002/joc.443610.7910/DVN/UI5LCE, 2018.