SnowModel Output Files

Author: Anna Grunes

Supplement for Grunes et al., 2025: “Development and Evaluation of a Physics-Based Snow Model to Assess Spatiotemporal Snowpack and Snowmelt Runoff Variability in the Northeastern U.S.A.”

Directories:

- SnowModel_Improvements

  - NetCDF files for all model runs used for SnowModel improvement study

- ClimateSensitivityAnalysis

  - NetCDF files for all model runs used for the climate sensitivity analysis

File Naming Convention:

‘sm_{start year}_{end year}_{variable name}_Run{run name}.nc’

i.e. ‘sm_22_23_SNOD_RunLW1.nc’ is the model output for 2022-2023 for snow depth with the model output for the LW1 run, which is the first liquid water percolation formulation test (see Table 1).

Table 1 shows the model run name information for the SnowModel Improvements directory.

Table 2 shows the model run name information for the Climate Sensitivity Analysis directory.

Table 1: Model calibration stepwise process to assess performance improvements from changes in rain-snow threshold and liquid water percolation routines. Each row represents a model run with the respective Run ID; each column shows the function used for each model framework or parameterization.

|  | Model Run Title | Run ID | Rain-Snow Parameterization | Liquid Water Parameterization | Other Parameters |
| --- | --- | --- | --- | --- | --- |
|  | Base Default | BD | Hyperbolic Tangent Function (Equation 1; Dai, 2008) | Maximum density threshold method (max density = 550 kg/m3) | Densification parameter = 5 |
|  |  |  |  |  | Lapse Rates = default |
| Rain-Snow Method Adjustment | Rain-Snow Default/Optimized Base Default | RS1 | Hyperbolic Tangent Function Constants: a, b, c, and d were set to −0.482292, 0.7205, 1.1662, and 1.0223, respectively. Mixed precipitation = Yes (Equation 1; Dai, 2008) | Maximum density threshold method (max density = 550 kg/m3) | Densification parameter = 0.4 Lapse Rates = S2S-derived (Table S1) |
|  | T-RH | RS2 | Temperature-RH Bivariate Function Constants: α, β, and γ were set to -10.04, 1.41, and 0.09, respectively. Mixed precipitation = No (Equation 2; Jennings et al., 2018) |  |  |
|  | T-wet bulb | RS3 | Wet Bulb Temperature Threshold Mixed precipitation = No (Equation S1) |  |  |
| Liquid Water Percolation Method Adjustment | Liquid Water Default | LW1 | Wet Bulb Temperature Threshold (Equation S1) | Maximum density threshold method max density = 550 kg/m3 n layers = 1 |  |
|  | Max Density (decreased) | LW2 |  | Maximum density threshold method (max density = 330 kg/m3) n layers = 1 |  |
|  | Gravity Drainage | LW3 |  | Gravity drainage method (Pflug et al., 2019) n layers = 6 |  |

Table 2: Climate sensitivity analysis model run IDs. Each ID represents a scenario of varying temperature and precipitation perturbations.

|  |  | Precipitation Perturbations |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | +0% | +10% | +20% | +30% |
| Temperature Perturbations | +0°C | CTL | CS_P_1 | CS_P_2 | CS_P_3 |
|  | +1.1°C | CS_T_1 | CS_P1_T1 | CS_P2_T1 | CS_P3_T1 |
|  | +1.6°C | CS_T_2 | CS_P1_T2 | CS_P2_T2 | CS_P3_T2 |
|  | +2.2°C | CS_T_3 | CS_P1_T3 | CS_P2_T3 | CS_P3_T3 |
|  | +2.7°C | CS_T_4 | CS_P1_T4 | CS_P2_T4 | CS_P3_T4 |
|  | 3.333 | CS_T_5 | CS_P1_T5 | CS_P2_T5 | CS_P3_T5 |